CPE301 – SPRING 2019

# Design Assignment 2A

Student Name: Tanner Tindall

Student #: 8000733733

Student Email: tindat1@unlv.nevada.edu

Primary Github address: https://github.com/TannerTindall51

Directory: https://github.com/TannerTindall51/tindalltannerm_submission/tree/master/Design_Assignments/DA2A

Design Assignment 2A: The goal of the assignment is use GPIO and delays:

1. Design a delay subroutine to generate a waveform on PORTB.3 with 55% DC and 0.75 sec period.

2. Connect a switch to PORTC.3 (active high - turn on the pull up transistor) to poll for an event to turn on the led at PORTB.2 for 2 sec after the event.

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- Atmel Studio 7.0 (Assembler, Simulator, & Debugger)

- Atmega328PB-Xmini

- PC Multi-Function Shield

- Logic Analyzer

- Switches

- LEDs

Ports used in 2A

Ports used in 2B

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

1A Assembly:

;

;Design Assignment 2A ASM

;Tanner Tindall

;

ldi  r19, 28	;initializing register values

ldi  r20, 101		;for delay loops below

ldi  r21, 233

ldi  r16, 34

ldi  r17, 124

ldi  r18, 86

sbi DDRB, 3

Duty_Cycle:			;primary loop

sbi PORTB, 3		;sets portB.3 high

call delay1		;delay for 337.5ms

cbi PORTB, 3		;clears portB setting it low

call delay2 		;delay for 412.5ms

jmp Duty_Cycle	;jump back to primary loop and restart

delay1: 			;delay1 = 337.5ms

dec  r21

brne delay1

dec  r20

brne delay1

dec  r19

brne delay1

nop

ret

delay2: 			;delay2 = 412.5ms

dec  r18

brne delay2

dec  r17

brne delay2

dec  r16

brne delay2

ret

1A C:

//

//Design Assignment 2A C

//Tanner Tindall

//

#include <avr/io.h>

#define F_CPU 16000000UL

#include <util/delay.h>

int main(void)

{

DDRB = 0xFF;				//sets all ports to output

PORTB = 0b00001000;		//sets portB.3 high

while (1)

{

PORTB = 0b00001000;	//sets portB.3 high, LED on

_delay_ms(337.5);		//delay for 337.5ms

PORTB = 0x00;			//sets portB low, LED off

_delay_ms(412.5);		//delay for 412.5ms

}

return 1;

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

1B Assembly:

;Design Assignment 2B ASM

;Tanner Tindall

.org 0x00

ldi  r16, 28		;initializing register values

ldi  r17, 101		;for delay loops below

ldi  r18, 233

ldi  r19, 34

ldi  r20, 124

ldi  r21, 86

ldi  r22, 163

ldi  r23, 87

ldi  r24, 3

ldi R25, 0xFF 		;loading all 1's into r25

out DDRB, r25 		;portB is set to output sonce r25 = 0xFF

out PORTB, r25 		;intiliazes all LED's to turn off

ldi r25, 0x00 		;loading 0 into r25 for next step

out DDRC, r25 		;sets the switches to inputs

Duty_Cycle:		;primary loop

cbi PORTB, 3		;turn on port needed

delay1: 			;delay1 = 337.5ms

dec  r18

brne delay1

dec  r17

brne delay1

dec  r16

brne delay1

nop

sbi PORTB, 3 		;turns off port

delay2: 			;delay2 = 412.5ms

dec  r21

brne delay2

dec  r20

brne delay2

dec  r19

brne delay2

sbis PINC, 3		;if button is pressed, then continue otherwise skip next instruction

rjmp Button_Press	;go to button_press loop

rjmp Duty_Cycle	;otherwise jump back to top and repeat

Button_Press:

cbi PORTB, 2		;turns on LED D4

delay3: 			;delay3 = 2s

dec  r24

brne delay3

dec  r23

brne delay3

dec  r22

brne delay3

nop

sbi PORTB, 2		;turns off LED D4

rjmp Duty_Cycle	;go back to primary loop and start over

2B C:

//Design Assignment 2B C

//Tanner Tindall

#include <avr/io.h>

#define F_CPU 16000000UL

#include <util/delay.h>

int main(void)

{

DDRB = 0xFF;	//sets all to output

PORTB = 0xFF;	//sets all to output

DDRC = 0x00;	//sets all to input

while (1)

{

PORTB |= (1<<3);	//turns on led 3

_delay_ms(337.5);	//delay for 337.5ms

PORTB &= ~(1<<3);	//turns off led 3

_delay_ms(412.5);	//delay for 412.5ms

if(!(PINC & (1<<3)))	//enters loop if button is pressed

{

PORTB |= (1<<3);

PORTB &= ~(1<<2);

_delay_ms(2000);

PORTB |= (1<<2);

}

}

return 0;

}

- SCHEMATICS

Obtained using Fritzing, shows the schematic for Task 1A/B

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Delay Loop 1 = 337.5ms

Delay Loop 2 = 412.5ms

Delay Loop 3 = 2s

337.5ms + 412.5ms = 750ms

Therefore, these 3 delay loops are responsible for producing a duty cycle of 55% in the given time period.

The image below displays the output waveform of the logic analyzer displaying the length (in time) of one period (750ms).

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

The images below show the proper LED in both the “ON” and “OFF” phase of the duty cycle.

- VIDEO LINKS OF EACH DEMO

Assembly Program Demo Video:  https://youtu.be/DzeufDcAsrQ

C program Demo Video: https://youtu.be/HES49L17CvM

- GITHUB LINK OF THIS DA

https://github.com/TannerTindall51/tindalltannerm_submission/tree/master/Design_Assignments/DA2A

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Tanner Tindall