CPE301 – SPRING 2019

# Design Assignment 4B

Student Name: Tanner Tindall

Student #: 8000733733

Student Email: tindat1@unlv.nevada.edu

Primary Github address: https://github.com/TannerTindall51

Directory:

https://github.com/TannerTindall51/tindalltannerm_submission/tree/master/Design_Assignments/DA4A

1. Write an AVR C program to control the speed of the DC Motor using a potentiometer connected to PC0. Use an interrupt on a button (PC1/2/3) to stop and start the motor at each click. The minimum speed of the motor should be 0 when pot is at minimum position and at maximum position should be 95% of PWM value.

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- Atmel Studio 7.0 (Assembler, Simulator, & Debugger)

- Atmega328PB-Xmini

- Micro USB

- LM35 Temperature Sensor

- Multi-Functional Shield

- MD08A

- DC Motor

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

//

//CPE301 - DA4A

//Tanner Tindall

//

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <stdbool.h>

#include <util/delay.h>

volatile unsigned int dutyCycle;	//value of duty cycle (changed with potentiometer)

volatile bool state = false;		//determines the state of motor

void configPWM()

{

TCCR0A = 0b10000011;			//set noninverting, fast PWM

TCCR0B = 0b00000011;			//set prescalar 1024

}

void startADC()

{

ADMUX = (1<<REFS0);								//set Vref and set ADC0 as input

ADCSRA = (1<<ADEN) | (1<<ADSC) | (1<<ADATE) | (1<<ADPS2) | (1<<ADPS1) | (1<<ADPS0);

}													//enable ADC and set prescalar to 128

void configDC()

{

dutyCycle = (adcRead()/1023.0) * 95;	//take value read from ADC, prescale it then take 95% of it

OCR0A = (dutyCycle/100.0)*255;		//convert dutyCycle into corresponding 8 bit value

}

int adcRead()

{

ADCSRA |= (1<<6);				//enable ADC conversion

while (!(ADCSRA & (1<<4)));		//waits for conversion to complete by ADIF flag

ADCSRA |= (1<<4);				//reset when complete

return ADC;						//return value once complete

}

int main(void)

{

DDRB |= (1<<2);				//set DDRB to output

DDRC &= ~(1<<PINC1|1<<PINC0);//set DDRC TO input

DDRD |= (1<<PIND6); 		//set DDRD to output (PWM)

PORTB |= (1<<2);			//set PB1 to HIGH

PORTC |= (0<<PINC0) | (1<<PINC1);//set PC0 to input (potentiometer) & PC1 to output (interrupt)

PCICR = (1<<PCIE1);			//enable pin interrupt

PCMSK1 = (1<<PCINT9);		//set interrupt on PC1

sei();						//enable interrupts

configPWM();				//config PWM

startADC();			//initialize ADC

while (1)

{

configDC();			//set duty cycle to corresponding potentiometer value

}

}

ISR(PCINT1_vect)				//PC1 interrupt subroutine

{

if(!(PINC & (1 << PINC1)))	//if button is pressed begin

{

if(state == false)		//if the DC motor is off

{

TCCR0B = 0x00;		// Turns off timer.

DDRD &= ~(1<<PIND6);

}

else if(state == true)	//if the DC is on

{

TCCR0B = 0b00000011;			//set prescalar 1024

DDRD |= (1<<PIND6);

}

PCIFR = (1<<PCIF1);		//clear interrupt

state = !state;			//invert current state

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

N/A

- SCHEMATICS

Note: This is the same schematic used in Dr. Venki’s annotations and is how the code this was configured to work.

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

N/A

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

Demo Video: https://youtu.be/LhFxsiOmJ7k

- GITHUB LINK OF THIS DA

https://github.com/TannerTindall51/tindalltannerm_submission/tree/master/Design_Assignments/DA4A

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Tanner Tindall