CPE301 – SPRING 2019

# Design Assignment 5A

Student Name: Tanner Tindall

Student #: 8000733733

Student Email: tindat1@unlv.nevada.edu

Primary Github address: https://github.com/TannerTindall51

Directory:

https://github.com/TannerTindall51/tindalltannerm_submission/tree/master/Design_Assignments/DA5A

Design Assignment 5 The goal of the assignment is to develop the above code to do the following:

1. Using the Multi-function shield, using a custom serial transfer protocol display the temperature sensor value (int only) of LM34/35 on to the seven-segment display.

2. Using the Multi-function shield, using the SPI protocol display the temperature sensor value (int only) of LM34/35 on to the seven-segment display.

3. Using one-wire interface display the temperature sensor for DS18B20 on the UART terminal.

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- Atmel Studio 7.0 (Assembler, Simulator, & Debugger)

- Atmega328PB-Xmini

- Micro USB

- Shield

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

//

//CPE301 - DA5A Task 1

//Tanner Tindall

//

#define BAUD 9600

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/setbaud.h>

#include <stdlib.h>

#include <stdio.h>

#include <util/delay.h>

#define latch 4

#define clock 7

#define data 0

const uint8_t SEGMENT_MAP[] = {0xC0,0xF9,0xA4,0xB0,0x99,0x92,0x82,0xF8,0X80,0X90};

const uint8_tSEGMENT_SELECT[] = {0xF1,0xF2,0xF4,0xF8};

volatile uint16_t tempC_value;

void shift_out_init(void)

{

DDRB |= (1<<data);

DDRD|=(1<<clock)|(1<<latch); //PD7 = SCLK & PD4 = latch

}

uint16_t adcRead()

{

ADCSRA |= (1<<6);				//enable ADC conversion

while (!(ADCSRA & (1<<latch)));	//waits for conversion to complete by ADIF flag

ADCSRA |= (1<<latch);			//reset when complete

return ADC;						//return value once complete

}

void startADC()

{

DDRC &= ~(1<<4);												//set PC4 to input

ADMUX = (1<<REFS1)| (1<<REFS0)|	(1<<MUX2);						//set Vref and set ADC4 as input

ADCSRA = (1<<ADEN) | (1<<ADPS2) | (1<<ADPS1) |	(1<<ADPS0);		//enable ADC and set prescalar to 128

ADCSRB = 0x00;													//set ADCSRB to free running

}

void shift_out(uint8_t indata)

{

for (uint8_t i = 0; i < 8; i++)

{

/* Write bit to data port. */

if (0 == (indata & _BV(7 - i)))

{

// digital_write(SHIFT_OUT_DATA, LOW);

PORTB &= (0 << data);

}

else

{

// digital_write(SHIFT_OUT_DATA, HIGH);

PORTB |= (1 << data);

}

/* Pulse clock to write next bit. */

PORTD |= (1<<clock);

PORTD &= (0<<clock);

}

}

uint16_t tempC()

{

uint16_t temperature = adcRead();				//record value from adc

tempC_value = ((temperature)*(110.0/1023.0));	//convert to temp value

shift_out(tempC_value);							//send the value out to display

}

int main(void)

{

shift_out_init();

startADC();

while (1)

{

adcRead();

tempC();

PORTD |= (1<<latch);

}

}

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 2/A

//

//CPE301 - DA5A Task 2

//Tanner Tindall

//

#define BAUD 9600

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/setbaud.h>

#include <stdlib.h>

#include <stdio.h>

#include <util/delay.h>

#define latch 4

const uint8_t segMapArray[]= {0xC0,0xF9,0xA4,0xB0,0x99,0x92,0x82,0xF8,0X80,0X90}; //taken from lect 13 summary

uint8_t segSelectArray[];

volatile int x;

volatile uint16_t tempC_value;

void startADC()

{

DDRC &= ~(1<<4);												//set PC4 to input

ADMUX = (1<<REFS1)| (1<<REFS0)|	(1<<MUX2);						//set Vref and set ADC4 as input

ADCSRA = (1<<ADEN) | (1<<ADPS2) | (1<<ADPS1) |	(1<<ADPS0);		//enable ADC and set prescalar to 128

ADCSRB = 0x00;													//set ADCSRB to free running

}

uint16_t adcRead()

{

ADCSRA |= (1<<6);				//enable ADC conversion

while (!(ADCSRA & (1<<latch)));	//waits for conversion to complete by ADIF flag

ADCSRA |= (1<<latch);			//reset when complete

return ADC;						//return value once complete

}

void initPorts()

{

DDRB |= (1<<3)|(1<<5);		//set MOSI and SCK output

DDRD |= (1<<latch);			//latch clock output

PORTD &= ~(1<<latch);		//initialize clock to 0

SPCR0 = 0x07;				//set SPI to master mode

}

void transmitSPI(uint8_t data)

{

SPDR0 = data;					//send data

while (!(SPSR0&(1<<SPIF)));		//wait until data register in emptied and all is sent

SPSR0 |= (1<<SPIF);				//reset flag

}

uint16_t tempC()

{

uint16_t temperature = adcRead();				//record value from adc

tempC_value = ((temperature)*(110.0/1023.0));	//convert to temp value

}

void Latch()

{

PORTD |= (1<<latch);		//set

_delay_ms(20);

PORTD &= ~(1<<latch);		//reset

}

int main()

{

initPorts();		//initialize ports

startADC();			//initialize adc

while(1)

{

tempC();		//collect a temp value from ADC

segConvert();	//convert to segment display and transmit

}

}

void segConvert()

{

for (x = 0; x < 4; x++)		//loop for each segment select

{

segSelectArray[x] = ((uint8_t)((tempC_value/pow(10, x))) % 10);	//scale temp value according to for loop

transmitSPI(segMapArray[segSelectArray[x]]);					//send the position value to defined array

transmitSPI(1<<(4+x));											//shift transmitted value

Latch();														//latch the clock before next iteration

}

}

- SCHEMATICS

N/A

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Task 1:

Task 2:

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Task 1:

Task 2:

- VIDEO LINKS OF EACH DEMO

Task 1: https://youtu.be/UloqvbBSXoI

Task 2: https://youtu.be/h4Y5UKhhAm8

- GITHUB LINK OF THIS DA

https://github.com/TannerTindall51/tindalltannerm_submission/tree/master/Design_Assignments/DA5A

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Tanner Tindall