CPE301 – SPRING 2019

# Design Assignment Midterm 2

Student Name: Tanner Tindall

Student #: 8000733733

Student Email: tindat1@unlv.nevada.edu

Primary Github address: https://github.com/TannerTindall51

Directory:

https://github.com/TannerTindall51/tindalltannerm_submission/tree/master/Midterm2

You’ll use the ADC, and PWM/CCP Module of the ATmega328/p to set and determine the speed of the DC Motor.

1. Use the motor driver, program the ATMega to drive the geared DC motor in CW and CCW direction for a given PWM.

2. Using the Potentiometer connected to ADC0, translate the ADC value to PWM value/ speed of the motor. Verify the operation.

3. Using the CCP capture pin of PWM1, in mode 1x and 2x determine the speed of the DC Motor for a set ADC Pot value/position.

4. Using CCP capture and interrupt (mode 4x), determine the speed of the DC Motor for a set ADC Pot value/position.

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- Atmel Studio 7.0 (Assembler, Simulator, & Debugger)

- Atmega328PB-Xmini

- Micro USB

- DC Motor

- Md80a Motor Driver

- Potentiometer

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

//

//CPE301 - Midterm 2

//Tanner Tindall

//

#define F_CPU 16000000UL

#define BAUD 9600

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/setbaud.h>

#include <stdlib.h>

#include <stdio.h>

#include <util/delay.h>

#include <stdbool.h>

#include <util/delay.h>

volatile uint16_t RPMx1_Value;

volatile uint16_t RPMx4_Value;

static char array[20];

volatile bool delay1;

volatile bool delay2;

void initializeUART()

{

DDRD |= 0x01;													//set transmit port direction

UBRR0 = 103;													//BAUD rate

UCSR0C |= (0<<UPM01) | (0<<UPM00) | (1<<UCSZ01) | (1<<UCSZ00);  //disable parity and set to asynchronous

}

void startADC()

{

DDRD &= ~(1<<0);												//set PD0 to input

ADMUX = (1<<REFS0);												//set Vref and set ADC0 as input

ADCSRA = (1<<ADEN) | (1<<ADATE)  | (1<<ADPS2) | (1<<ADPS1) |	(1<<ADPS0);		//enable ADC and set prescalar to 128

ADCSRB = (1<<ADTS0);											//set analog compare match

}

uint16_t adcRead()

{

ADCSRA |= (1<<6);				//enable ADC conversion

while (!(ADCSRA & (1<<4)));		//waits for conversion to complete by ADIF flag

ADCSRA |= (1<<4);				//reset when complete

return ADC;						//return value once complete

}

void USART_tx_transmit()

{

UBRR0 = 103;

UCSR0B |= (1 << TXCIE0) | (1<<TXEN0);		//enable data transmission

DDRD |= 0x01;								//transmit through PD1

}

void USART_tx(char*data)						//outputs data to terminal

{

while((*data != '\0'))						//loop until all data is emptied

{

while(!(UCSR0A & (1 << UDRE0)));		//wait until data register in emptied

UDR0 = *data;							//once emptied, import into data register UDR0

data++;									//increments pointer position for data

}

}

void adcValue()

{

uint16_t x = adcRead();

float adcVal = ((x)*(110.0/1023.0));

snprintf(array,sizeof(array), "%f\r\n", adcVal);

USART_tx(array);

return;

}

void setPorts()

{

DDRB |= (1<<3) | (1<<4);//set output ports used for motor direction

PORTB &= ~(1<<3);

PORTB |= (1<<4);

DDRD &= ~(1<<2);		//set INT0

PORTD &= ~(1<<2);

DDRD &= ~(1<<3);		//set INT1

PORTD &= ~(1<<3);

DDRD |= (1<<4);			//input capture 1

PORTD &= ~(1<<4);

DDRD |= (1<<5);			//input capture 2

PORTD &= ~(1<<5);

}

int main()

{

startADC();				//initialize functions above

USART_tx_transmit();

initializeUART();

initializeTimer0();

initializeTimer4();

setPorts();

sei();

while(1)				//display RPMs to UART

{

USART_tx("RPM:");

RPMx1();

USART_tx("\n");

USART_tx("RPM:");

RPMx2();

USART_tx("\n");

USART_tx("RPM:");

RPMx4();

USART_tx("\n");

_delay_ms(10);

}

}

void RPMx1()

{

initializeTimer3();				//run counter

initializeTimer4();				//run delay (100ms)

delay1 = true; 					//set flag for while loop below

while(delay1); 					//collect data until delay is finished

PORTD &= ~(1<<4); 				//reset interrupts

PORTD &= ~(1<<5);

return (float)(RPMx4_Value * (60.0/96.0)); //convert given value to rpm

}

void RPMx2()

{

initializeTimer3();				//run counter

initializeTimer4();				//run delay (100ms)

EIMSK |= (1<<INT1); 			//set INT1 to handle interrupt for data collection

EICRA |= (1<<ISC01) ; 			//enable external interrupt on falling edge

delay1 = true; 					//set flag for while loop below

while(delay1); 					//collect data until delay is finished

disableTrigger();				//disable external interrupt

PORTD &= ~(1<<4); 				//reset interrupts

PORTD &= ~(1<<5);

return (float)(RPMx4_Value * (60.0)/(192.0)); //convert given value to rpm

}

void RPMx4()

{

initializeTimer1();					//run counter 1

initializeTimer3();					//run counter 2

initializeTimer4();					//run delay (100ms)

EIMSK |= (1<<INT1) | (1<<INT0); 	//set INT0/1 to handle interrupt for data collection

EICRA |= (1<<ISC11) | (1<<ISC00); 	//enable external interrupt on falling edge

delay1 = true; 						//set flags

delay2 = true;

while(delay1 || delay2); 			//collect data until delay is finished

disableTrigger();					//disable external interrupt

PORTD &= ~(1<<4);					//reset interrupts

PORTD &= ~(1<<5);

return (float)((RPMx1_Value + RPMx4_Value) * (60.0)/(384.0)); //convert given value to rpm

}

void initializeTimer0() 						//PWM timer

{

TCCR0A |= (1<<COM0A1) | (1<<WGM00);			//non-inverting, Fast PWM

TCCR0B |= (1<<WGM02) | (1<<CS02);

TIMSK0 |= (1<<TOIE0);						//enable interrupts

TCNT0 = 0x00;

OCR0A = 0x00;

}

void initializeTimer1()											//RPMx4

{

DDRD &= ~(1<<7);

TCNT1 = 0x00;

TCCR1B |= (1<<FOC4B) | (1<<CS32) | (1<<CS31) | (1<<CS30);	//set CTC mode and prescaler of 64

TIMSK1 |= (1<<ICIE1);

}

void initializeTimer3()											//RPMx1 & RPMx2 & RPMx4

{

DDRD &= ~(1<<6);

TCNT3 = 0x00;												//initialize timer3

TCCR3B |= (1<<FOC4B) | (1<<CS32) | (1<<CS31) | (1<<CS30); 	//set to rising edge trigger

TIMSK3 |= (1<<ICIE3); 										//set input compare interrupt

}

void initializeTimer4()								//ADC sampling delay timer

{

TCNT4 = 0x00;  									//initialize timer

TCCR4B |= (1<<WGM42) | (1<<CS41) | (1<<CS40); 	//set CTC mode and prescaler of 64

TIMSK4 |= (1<<OCIE4A); 							//set output compare interrupt

OCR4A  = 24999; 								//100ms in cycles

}

ISR(INT0_vect)

{

motorDirection();			//motor changes direction on interrupt

if(delay1)					//if collecting data

{

TCCR3B ^= (CS31);		//switch edge trigger

}

}

ISR(INT1_vect)

{

if(delay2)					//if collecting data

{

TCCR1B ^= (CS11);		//switch edge trigger

}

}

ISR(TIMER0_OVF_vect)			//PWM interrupt subroutine

{

ADCSRA |= (1<<ADIE);		//set ADC interrupt

}

ISR(TIMER1_CAPT_vect)			//RPMx4 interrupt subroutine

{

TIMSK1 &= ~(1U<<ICIE1);		//reset interrupt

delay2 = false;				//change delay condition to enable calculations

RPMx1_Value = (float) ICR1;	//store value from input data

delay1 = false;				//change flag for calculation enable

}

ISR(TIMER3_CAPT_vect)			//RPMx1 & RPMx2 & RPMx4 interrupt subroutine

{

TIMSK3 &= ~(1<<ICIE3);		//reset interrupt

delay1 = false;				//change delay condition to enable calculations

RPMx4_Value = (float) ICR3;	//store value from input data

delay2 = false;				//change flag for calculation enable

}

ISR(TIMER4_COMPA_vect)

{

TIMSK3 &= ~(TOIE0);

PORTD |= (1<<4); 		//set interrupt

if(delay2)

{

PORTD |= (1<<5);	//set interrupt for additional timer

}

}

void disableTrigger()

{

EIMSK = 0x00;			//disable INT1 interrupt

EICRA = 0x00;			//disable external interrupt

}

void motorDirection()		//invert current direction of motor

{							//by reversing H-Bridge config

PORTB ^= (1<<3);

PORTB ^= (1<<4);

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

N/A

- SCHEMATICS

N/A

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

N/A

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

Demo Video:  https://youtu.be/6lWBPpY6V_Y

- GITHUB LINK OF THIS DA

https://github.com/TannerTindall51/tindalltannerm_submission/tree/master/Midterm2

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Tanner Tindall